TASK 04

1. Connect a USB webcam to your computer, install any required drivers, and verify that the webcam works by opening the default camera app and capturing a photo.

STEPS:

1. Connect the webcam: Plug the webcam's USB cable into an available USB port on the computer.

2. Install the drivers: Wait for Windows to detect the webcam and automatically install the necessary drivers. If it does not work, download the correct driver from the webcam manufacturer's official website.

3. Check Device Manager: Right-click the Start button, open Device Manager, and look under Cameras or Imaging devices to confirm that the webcam is detected.

4. Allow camera access: Go to Settings → Privacy & security → Camera in Windows 11 (or Settings → Privacy → Camera in Windows 10). Enable camera access and allow apps to use the camera.

5. Open the Camera app: Click Start, search for Camera, and open the Windows Camera app.

6. Test the webcam: Check that the live video appears on the screen. Adjust the webcam's position if necessary.

7. Capture a photo: Click the camera button in the app to take a picture.

8. Verify the photo: Open the saved photo, usually in the Pictures → Camera Roll folder, and confirm that the image is clear.

2. Set up a local printer (USB or network), install the correct driver, and print a test page to confirm successful installation.

STEPS:

1. Connect the printer: Connect the printer to the computer using a USB cable, or connect both the printer and computer to the same Wi-Fi network.

2. Turn on the printer: Connect the printer to a power source and switch it on. Ensure that paper is loaded and ink or toner is available.

3. Install the printer driver: Allow Windows to install the driver automatically. If required, download the correct driver for the printer model from the manufacturer's official website.

4. Add the printer: Open Settings → Bluetooth & devices → Printers & scanners in Windows 11 (or Settings → Devices → Printers & scanners in Windows 10). Select Add device and follow the instructions.

5. Set the default printer (optional): Select the installed printer and choose the option to set it as the default printer, if needed.

6. Print a test page: Open Printers & scanners, select your printer, open Printer properties, and click Print Test Page.

7. Verify the result: Check that the printer prints the test page correctly. Confirm that the text and graphics are clear and that there are no paper jams or error messages.

3. Simulate a connectivity issue by unplugging the printer's USB cable, then use Windows Device Manager to identify and resolve the 'printer not detected' problem.<br><br><em><strong>Hint:</strong> Check for error icons and use the 'Scan for hardware changes' option after reconnecting.</em>

STEPS:

1. Disconnect the printer: Turn on the computer and printer, then unplug the printer's USB cable from the computer. Windows may show that the printer is unavailable.

2. Open Device Manager: Right-click the Windows Start button and select Device Manager.

3. Check for errors: Expand Printers, Print queues, or Universal Serial Bus controllers. Look for a yellow warning icon or an unknown device. The disconnected printer may simply disappear from the list.

4. Reconnect the printer: Plug the USB cable securely back into the computer and ensure the printer is switched on.

5. Scan for hardware changes: In Device Manager, select Action → Scan for hardware changes. Windows will search for connected devices and attempt to detect the printer again.

6. Check the driver: If the printer appears with a warning icon, right-click it and select Update driver. Follow the instructions to install a suitable driver.

7. Verify the connection: Open Settings → Bluetooth & devices → Printers & scanners in Windows 11, or Settings → Devices → Printers & scanners in Windows 10. Confirm that the printer is listed.

8. Print a test page: Open the printer's properties and select Print Test Page to confirm that it is working.

4. A user reports that their webcam video is not displaying in Zoom, but it works in the default   camera app. Troubleshoot and resolve the issue so the webcam works in bothapplications.<br><br><em><strong>Hint:</strong> Check if another app is using the webcam or if Zoom permissions are enabled.</em>.

STEPS:

1. Close other camera applications: Close the Windows Camera app and any other applications that may be using the webcam, such as Microsoft Teams or Skype.

2. Check Zoom camera settings: Open Zoom and go to Settings → Video & effects (or Video). Select the correct webcam from the Camera dropdown menu.

3. Check camera permissions: Open Windows Settings → Privacy & security → Camera. Enable camera access and allow desktop apps to access the camera. In Windows 10, go to Settings → Privacy → Camera.

4. Restart Zoom: Completely close Zoom, reopen it, and join a test meeting to check whether the webcam video appears.

5. Check the webcam connection: If using a USB webcam, ensure the cable is connected securely. Try another USB port if necessary.

6. Update Zoom: Check for updates by opening Zoom and selecting your profile menu → Check for Updates.

7. Update the webcam driver: Right-click Start, open Device Manager, expand Cameras, right-click the webcam, and select Update driver.

8. Test the webcam again: Open Zoom's video settings and confirm that the webcam displays a live preview. Then open the Windows Camera app to ensure it still works there.